Azure BSCNE Capstone
Functionality Report

# Introduction

Provide a functionality report detailing the 8 test-case scenarios used to verify the utility of your network project. Six of the test-case scenarios must be from the provided predefined list, with the remaining two test cases created by you but aligned with the topic given in the test case title. The functionality report should be written so that a networking peer could replicate the steps for a successful test of your networking solution.

| Student Name | Christopher Matti |
| --- | --- |
| WGU Student ID | 011836844 |
| WGU Student Email | cmatti@wgu.edu |

# Test Case #1: Local Networks - Basic Network Segmentation at Layer 2 via VLANs and 802.1q

Your network traffic should be segmented per department or service function at Layer 2 to enhance security and reduce network congestion at the switching layer while allowing segmented traffic to traverse between switches (VLAN trunking). For each setup, describe your configuration choices and how you verified that the network segments are isolated and that any allowed communication works correctly.

# Functionality

Describe the functionality of the test case in relation to your network project. Identify the relevant tools (devices, subnets, etc.) used in this test case and their specific interactions.

The primary objective of this test case is to demonstrate secure Layer 2 segmentation within the on-premises environment. By implementing Virtual Local Area Networks (VLANs), we partition the broadcast domain into three distinct logical segments: VLAN 10 (Shared Services), VLAN 20 (First Floor PCs), and VLAN 30 (Second Floor PCs).

Traffic isolation is maintained at the data link layer, ensuring that sensitive management traffic (AD DS/DNS) is separated from general user traffic. Inter-VLAN communication is handled by the pfSense Firewall, which acts as the "router-on-a-stick," while 802.1q trunking allows multiple VLAN tags to traverse the single physical link between the Main Switch and the pfSense appliance.

Relevant Tools & Devices:

- pfSense Firewall: Acts as the DHCP server for all segments and handles inter-VLAN routing.

- Main Switch (VLAN 10): Manages the core infrastructure and provides the 802.1q trunk port.

- Workstation Switches (1 & 2): Access layer switches that assign physical ports to VLAN 20 and VLAN 30.

- Subnets: 10.0.1.0/24 (VLAN 10), 10.0.10.0/24 (VLAN 20), and 10.0.20.0/24 (VLAN 30).

## Network Diagram or Segment

Provide a network diagram or segment visualizing the topology and devices used in this test case.

The relevant segment for this test case is the On-Prem / Local Side of the architecture. It highlights the physical and logical connections between the pfSense Firewall, the Shared Services switch, and the two departmental workstation switches.

## Testing Method

Summarize the testing method used to verify functionality of the network project within the virtual lab environment, including any metrics of success.

The testing method focuses on validating Layer 2 isolation and Layer 3 reachability through controlled ICMP (ping) tests and DHCP verification.

- Isolation Verification: Ensure a device in VLAN 20 cannot communicate with a device in VLAN 30 unless specifically permitted by firewall rules.

- Trunking Verification: Confirm that devices on different switches can receive the correct IP address from the centralized pfSense DHCP server via the trunk link.

- Metrics of Success: 1. Successful DHCP IP assignment within the correct subnet range for each VLAN. 2. 0% packet loss for allowed inter-VLAN traffic (e.g., Workstations to Shared Services). 3. 100% packet loss for denied traffic (e.g., Guest/Unauthorized access to VLAN 10).

## Process List

Provide a comprehensive process list of the steps taken within the network project to run the testing method. Include screenshots to illustrate the process and ensure clarity for others attempting to replicate the test.

- VLAN Creation on pfSense:

- Navigate to Interfaces > Assignments > VLANs.

- Create VLAN tags 10, 20, and 30 on the parent LAN interface.

- Interface Configuration:

  - Assign the new VLAN tags to logical interfaces and enable them with appropriate static IPv4 addresses (e.g., 10.0.10.1 for VLAN 20).

- Switch Trunking Setup:

- Configure the uplink port on the Main Switch as a Trunk Port (Tagged) to allow VLANs 10, 20, and 30.

- Configure the ports connecting the Workstation Switches as tagged or untagged based on the hardware requirements (802.1q).

- Access Port Assignment:

- Set ports on Workstation Switch 1 to Access Mode for VLAN 20.

- Set ports on Workstation Switch 2 to Access Mode for VLAN 30.

- DHCP Scope Implementation:

- In pfSense, enable DHCP Server for each VLAN interface and define the specific range (e.g., 10.0.10.100 to 10.0.10.200 for VLAN 20).

- Verification (Client Side):

- Renew the IP address on User PC 1 (VLAN 20) and User PC 1 (VLAN 30) using ipconfig /renew.

- Perform a ping from VLAN 20 to the AD DS Server in VLAN 10 to confirm the trunk and routing are functional.

- Perform a ping from VLAN 20 to a device in VLAN 30 to confirm isolation (based on existing firewall rules).

## Screenshots:

VLAN config

VLAN 20 DHCP DHCP Config:

VLAN 20 DHCP DHCP Config:

Main Switch Config:

First Floor Switch config

Second Floor Switch Config

# Test Case #2: Local Networks - Accessing External Resources—Routing and Traffic Security

User devices on your network should have dynamic addresses that are assigned through DHCP unless they provide a service that requires a static address. You must also have at least one network resource that requires a static address.

# Functionality

Describe the functionality of the test case in relation to your network project. Identify the relevant tools (devices, subnets, etc.) used in this test case and their specific interactions.

## Network Diagram or Segment

Provide a network diagram or segment visualizing the topology and devices used in this test case.

## Testing Method

Summarize the testing method used to verify functionality of the network project within the virtual lab environment, including any metrics of success.

## Process List

Provide a comprehensive process list of the steps taken within the network project to run the testing method. Include screenshots to illustrate the process and ensure clarity for others attempting to replicate the test.

# Test Case #3: Local Networks – Device Discovery and Reachability

Your network solution must include multiple network segments with access controls that allow traffic from a device on one network to access the resources of a device on another network. Similarly, there must be devices on one network that cannot access resources on a different network.

## Functionality

Describe the functionality of the test case in relation to your network project. Identify the relevant tools (devices, subnets, etc.) used in this test case and their specific interactions.

## Network Diagram or Segment

Provide a network diagram or segment visualizing the topology and devices used in this test case.

## Testing Method

Summarize the testing method used to verify functionality of the network project within the virtual lab environment, including any metrics of success.

## Process List

Provide a comprehensive process list of the steps taken within the network project to run the testing method. Include screenshots to illustrate the process and ensure clarity for others attempting to replicate the test.

# Test Case #4: Cloud Networks - Network Segmentation

Your cloud network traffic (e.g., VPC, VNet, or similar) should be segmented using subnets, virtual networks, or equivalent constructs to enhance security and reduce congestion. You must ensure segmented traffic can traverse between subnets or virtual networks as needed (e.g., cloud routing tables or equivlent). Within your chosen cloud platform (Azure), your network traffic should be segmented using platform-specific components.

# Functionality

Describe the functionality of the test case in relation to your network project. Identify the relevant tools (devices, subnets, etc.) used in this test case and their specific interactions.

## Network Diagram or Segment

Provide a network diagram or segment visualizing the topology and devices used in this test case.

## Testing Method

Summarize the testing method used to verify functionality of the network project within the virtual lab environment, including any metrics of success.

## Process List

Provide a comprehensive process list of the steps taken within the network project to run the testing method. Include screenshots to illustrate the process and ensure clarity for others attempting to replicate the test.

# Test Case #5: Cloud Networks – Accessing External Resources-Routing and Traffic Security

Your cloud networks should typically use dynamically assigned IP addressing from the cloud platform unless they provide a service that requires a static IP address. You must also have at least one network resource that requires a static address.

## Functionality

Describe the functionality of the test case in relation to your network project. Identify the relevant tools (devices, subnets, etc.) used in this test case and their specific interactions.

## Network Diagram or Segment

Provide a network diagram or segment visualizing the topology and devices used in this test case.

## Testing Method

Summarize the testing method used to verify functionality of the network project within the virtual lab environment, including any metrics of success.

## Process List

Provide a comprehensive process list of the steps taken within the network project to run the testing method. Include screenshots to illustrate the process and ensure clarity for others attempting to replicate the test.

# Test Case #6: Cloud Networks – Device Discovery and Reachability

Your network solution must include multiple network segments within the same VPC or VNet with access controls that allow traffic from a device on one network to access the resources of a device on another network. Similarly, there must be devices on one network that cannot access resources on a different network.

# Functionality

Describe the functionality of the test case in relation to your network project. Identify the relevant tools (devices, subnets, etc.) used in this test case and their specific interactions.

## Network Diagram or Segment

Provide a network diagram or segment visualizing the topology and devices used in this test case.

## Testing Method

Summarize the testing method used to verify functionality of the network project within the virtual lab environment, including any metrics of success.

## Process List

Provide a comprehensive process list of the steps taken within the network project to run the testing method. Include screenshots to illustrate the process and ensure clarity for others attempting to replicate the test.

# Test Case #7: VPN: Bridging the Networks

## Custom Test Case

Define a custom test case to be run within your network project aligned to your specific organizational need or opportunity identified in Task 1. The custom test should be equivalent in scope and requirements to the predefined test cases and pertain to VPN connectivity between local and cloud networks.

# Functionality

Describe the functionality of the test case in relation to your network project. Identify the relevant tools (devices, subnets, etc.) used in this test case and their specific interactions.

## Network Diagram or Segment

Provide a network diagram or segment visualizing the topology and devices used in this test case.

## Testing Method

Summarize the testing method used to verify functionality of the network project within the virtual lab environment, including any metrics of success.

## Process List

Provide a comprehensive process list of the steps taken within the network project to run the testing method. Include screenshots to illustrate the process and ensure clarity for others attempting to replicate the test.

# Test Case #8: Hybrid Network Security

## Custom Test Case

Define a custom test case to be run within your network project aligned to your specific organizational need or opportunity identified in Task 1. The custom test case should be equivalent in scope and requirements to the predefined test cases and pertain hybrid network security.

## Functionality

Describe the functionality of the test case in relation to your network project. Identify the relevant tools (devices, subnets, etc.) used in this test case and their specific interactions.

## Network Diagram or Segment

Provide a network diagram or segment visualizing the topology and devices used in this test case.

## Testing Method

Summarize the testing method used to verify functionality of the network project within the virtual lab environment, including any metrics of success.

## Process List

Provide a comprehensive process list of the steps taken within the network project to run the testing method. Include screenshots to illustrate the process and ensure clarity for others attempting to replicate the test.

# Network Troubleshooting

Discuss how you analyzed the network to identify, troubleshoot, and resolve issues during development or when ensuring functionality of the test cases.